SITE survey-template 4

| Project Address | ADDRESS |
| --- | --- |

| Market Business Unit | MARKET |
| --- | --- |

| Project Name | PROJECT NAME |
| --- | --- |
| Project Number Project Type | PROJECT # Tenant Improvement, Major Renovation |
| THIS TEMPLATE COVERS THE FOLLOWING PROJECT TYPES. Licensed Owned; Company Owned. New Store or Scrape and Replace; Conversion; ☐. w/ interior work only; ☐; Ground Up; ☐. w/ interior and exterior work; ☐; Tenant Improvement. Renovation; w/ interior work Only; ☐. w/ interior work only; ☐; w/ interior and exterior work; ☐. w/ interior and exterior work; ☐; Major Renovations; ☐. Minor Renovation; ☐. New Kiosk; ☐. |  |
| Prepared by | Change Up, Inc. |  |
| Date | DATE |  |
| Survey Template | Version Q3FY21 |  |

Template Change Log

| Revision | Description of Changes | Approved by |
| --- | --- | --- |
| Q4FY19 | Removed minor renovations from project types on template cover page. 2.1 - Added drawings requirements for Health permit 2.4 – Added Grease interceptor requirements and variance questions 2.8 – Added questions regarding AHJ approved plumbing fixtures 4.6 – Revised Telephone and broadband requirements to ensure pertinent information is collected for project scoping | CE |
| Q3FY20 | 2.1 – Added Reference link to Starbucks equipment cut sheet binder 2.4 – Removed variance request information from this section and added to section 4.5 4.5 – Made revisions to this section to support Starbucks new “Grease Interceptor purchasing program” on the SDRC. | CE |
| Q2FY21 | 4.6 – Made revisions to this section to support Starbucks updated Site Internet & Voice Systems process | CE |
| Q3FY21 | 2.2 – Health: revision to permit overview call out potential need for preview of metal framework casework with AHJ 2.8 – Revisions to this section highlighting the potential need for early conversations with AHJ around Starbucks metal framework casework system | MB |

Note: All Revisions are highlighted in red below

## TABLE OF CONTENTS

- Refer to the Instructions document for guidance on conducting a Site Survey and preparing this report.  Sections are to be completed as applicable and appropriate for each project site and scope.

FINDINGS SUMMARY	2

1. PROJECT DATA	6

2. BUILDING CODES & PERMITS	10

3. ZONING & SITE INFORMATION	20

4. UTILITIES	22

5. BUILDING INFORMATION	30

6. FACILITY/JOB SITE MANAGEMENT	34

# FINDINGS SUMMARY

The following is a summary of the most significant issues and findings affecting this project.  The following may have significant impact on project Scope, Schedule, and/or Budget.

Note: These items are also highlighted in orange as part of the main report.

## CRITICAL FINDINGS – (CLICK ARROW IN LEFT HAND MARGINE TO DISPLAY)

| 2. Building / Planning / Health |  |
| --- | --- |
| Does Building Require Sprinklers? | ☐ Yes ☐ No (If Yes See Sections 1.1) A breakdown of square footage has not been provided; therefore, an accurate calculation cannot be completed. If the occupant load is below 50 then the space can be considered B occupancy. If the occupant load is above 50 then the space can be considered A-2 occupancy. If the occupancy load is below 100 sprinklers will not be required. 903.2.1.2 2018 International Building Code It is likely that the occupancy load will be less than 100 once the final tenant fit out is complete and the occupancy load calculation is completed by the AOR. |
| Does the jurisdiction have an expedited review process? | ☐ Yes ☐ No (If Yes See Sections 2.3a) |
| Are there any Fire separation requirements | ☒ Yes ☐ No (If See Sections 2.5) |
| Are there an accessible path of Travel restrictions? | ☐ Yes ☐ No (If No See Sections 2.6) Not applicable to this report. Due diligence report only. |
| Are the existing restrooms handicap accessible? | ☐ Yes ☐ No (If No See Sections 2.7) Not applicable to this report. Due diligence report only. |
| Is a hot water Sanitizing Dishwasher required? | ☐ Yes ☒ No (If Yes See Sections 2.8) |
| Are special base of wall details required? | ☒ Yes ☐ No (If Yes See Sections 2.8) When water flush cleaning methods are used in a food establishment, floors shall have graded drains, and floor and wall junctures must be coved and sealed. When other cleaning methods are used besides water-flushing, the floor and wall junctures shall be coved and closed with gaps no wider than 1/32”. Check for additional specifications in the local code. |
| Case work Base Requirement | ☒ Yes ☐ No (If Yes See Sections 2.8) 6-inch legs |
| Are sneeze guards required? | ☒ Yes ☐ No (If Yes See Sections 2.8) Any food on display shall be protected from contamination through the use of food/sneeze guards and display cases. Food shields are to be used as a barrier between the consumer and the food items. Check for additional specifications in the local code. |
| Are there remote storage Requirements? | ☒ Yes ☐ No (If Yes See Sections 2.8) Food shall be protected from contamination by being stored in a clean, dry location where it will not be exposed to dust, splash, or other contaminants. Food containers shall be stored at least 15 cm above the ground. Food shall never be stored in locker rooms, toilet rooms, dressing rooms, garbage rooms, under unshielded sewer lines, under leaking water lines, under open stairwells, near contaminants, or in unapproved exterior storage locations. Check for additional specifications in the local code. |
| Is outdoor seating allowed? | ☒ Yes ☐ No (If No See Sections 2.10) |
| 3. Zoning and Site |  |
| Are there any signage restrictions? | ☐ Yes ☐ No (If Yes See Sections 3.7) |
| Does the Drive Thru (DT) accommodate a 4 to 5 car stack? | ☒ Yes ☐ No (If No See Sections 3.11) |
| 4. Utilities |  |
| Do the RTU’s need to be replaced, relocated, or upgraded? | ☐ Yes ☐ No (If yes See Sections 4.1) Not applicable to this report. Due diligence report only. |
| Water Heater: Does the existing heater need to be relocated / replaced? | ☐ Yes ☐ No (If yes See Sections 4.1) Not applicable to this report. Due diligence report only. |
| Is a 120-208V, 3-phase, 4 wire, and 400 amp service available? (This is the min. service Req.) | ☐ Yes ☐ No (If no See Sections 4.2) Not applicable to this report. Due diligence report only. |
| Does the existing water main need to be upgraded? | ☐ Yes ☐ No (If yes See Sections 4.3) Not applicable to this report. Due diligence report only. |
| Does the building require sprinklers? | ☐ Yes ☐ No (If yes See Sections 4.9) A breakdown of square footage has not been provided; therefore, an accurate calculation cannot be completed. If the occupant load is below 50 then the space can be considered B occupancy. If the occupant load is above 50 then the space can be considered A-2 occupancy. If the occupancy load is below 100 sprinklers will not be required. 903.2.1.2 2018 International Building Code It is likely that the occupancy load will be less than 100 once the final tenant fit out is complete and the occupancy load calculation is completed by the AOR. |
| Will the (e) fire sprinkler need modifications? | ☐ Yes ☐ No (If yes See Sections 4.9) Dependent on design. |
| Is a fire alarm panel required? | ☐ Yes ☒ No (If yes See Sections 4.9) |
| 5. Building |  |
| Will the (e) roof need to be replaced? | ☐ Yes ☐ No (If yes See Sections 5.5) Not applicable to this report. Due diligence report only. |
| Are structural modifications recommended/required for the roof? | ☐ Yes ☐ No (If yes See Sections 5.5) Not applicable to this report. Due diligence report only. |
| Are structural modifications recommended or required for the floor? | ☐ Yes ☐ No (If yes See Sections 5.9) Not applicable to this report. Due diligence report only. |
| Are Fixtures compliant w/ ADA standards | ☐ Yes ☐ No (If no See Sections 5.13) Not applicable to this report. Due diligence report only. |
| Is there visible water damage in space? | ☐ Yes ☐ No (If yes See Sections 5.15) Not applicable to this report. Due diligence report only. |
| Is there any evidence of asbestos being present? | ☐ Yes ☐ No (If yes See Sections 5.15) Not applicable to this report. Due diligence report only. |

Additional details about each of these issues can be found in the body of the report.

# 1. PROJECT DATA

## 1.1 General Information

| TBD | Project Description | ☐ New Construction |  | ☐ STB Single Tenant Building |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ☐ Existing Construction |  | ☐ As is Conversion ☐ Tenant Imprv. |  |  | ☐ Drive Thru |  |
|  |  | ☐ Renovation |  | ☐ Major ☐ Minor |  |  | ☐ Drive Thru |  |
| ☐ | Overall Building Area (typically determined by Site Surveyor using field measurements) |  | Click or tap here to enter text. |  |  | sqft |  |  |
| ☐ | Tenant Area (gross sf) (Total area available for lease) |  | Click or tap here to enter text. |  |  | sqft |  |  |
| ☐ | Tenant Area (net sf) (Total area to be used for the store build-out) |  | Click or tap here to enter text. |  |  | sqft |  |  |
| ☐ | Common Area (Outdoor Patio / Kiosk Seating) (Area that is not part of the lease) |  | Click or tap here to enter text. |  |  | sqft |  |  |
| ☐ | Zoning Type | Click or tap here to enter text. | Construction Type |  | (Assumed) | Occupancy |  | (Assumed) |
| ☐ | Int. Seating # |  | Unable to determine. Due diligence report only. |  |  |  |  |  |
| ☐ | Does Building Require Sprinklers? |  | ☐ Yes ☐ No A breakdown of square footage has not been provided; therefore, an accurate calculation cannot be completed. If the occupant load is below 50 then the space can be considered B occupancy. If the occupant load is above 50 then the space can be considered A-2 occupancy. If the occupancy load is below 100 sprinklers will not be required. 903.2.1.2 2018 International Building Code It is likely that the occupancy load will be less than 100 once the final tenant fit out is complete and the occupancy load calculation is completed by the AOR. |  |  |  |  |  |
|  | Other |  | Describe |  |  |  |  |  |

## 1.2 Architectural & Site Context – NA

## 1.3 Location Maps

- Map(s) to show surrounding context of city/streets/neighborhood.

- Highlight emergency response service locations within a one (1) mile (1.6 km) radius (Hospital, Fire Department, Police Station, etc.)

- If Evenings program, identify any nightclubs, housing, schools, churches, alcohol treatment centers, or marijuana dispensaries within a one (1) mile radius.

- Show True North

Area Map

Location Map

Hospital

Fire Department zzzz

Police

Emergency Locations Map

## 1.4 Contacts

- The following contacts may also be applicable to your project: Real Estate Broker/Agent, Zoning Department, Historic Review, Traffic, Gas, etc.  Insert additional contacts in the table below as appropriate.

| Contact | Name, Title | Address, City, State, Zip | Phone, Fax, E-mail, website |
| --- | --- | --- | --- |
| Starbucks Contact Design Manager Construction Manager | Not applicable to this report. Due diligence report only. | Not applicable to this report. Due diligence report only. | Not applicable to this report. Due diligence report only. |
| Surveyor/Architect |  |  | Ph: Email: Website: |
| Landlord Contact |  |  | Ph: Email: Website: |
| Building Department |  |  | Ph: Email: Website: |
| Fire Department |  |  | Ph: Email: Website: |
| Electric |  |  | Ph: Email: Website: |
| Cable/Telephone | Multiple providers. | Multiple providers. | Multiple providers. |
| Water |  |  | Ph: Email: Website: |
| Sewer |  |  | Ph: Email: Website: |
| Health Department |  |  | Ph: Email: Website: |

## Licensed Stores

| Contact | Name, Title | Address, City, State, Zip | Phone, Fax, E-mail, website |
| --- | --- | --- | --- |
| Licensee Contact | Not applicable to this report. Due diligence report only. | Not applicable to this report. Due diligence report only. | Not applicable to this report. Due diligence report only. |
| Licensee Architect | Not applicable to this report. Due diligence report only. | Not applicable to this report. Due diligence report only. | Not applicable to this report. Due diligence report only. |

# 2. BUILDING CODES & PERMITS

## 2.1 Building Codes

| TBD | Codes |  |
| --- | --- | --- |
| ☐ | What are the applicable Building Codes? | 2015 International Building Code 2015 International Residential Code 2015 International Plumbing Code 2015 International Mechanical Code 2017 National Electric Code NFPA 70 2015 International Energy Conservation Code 2015 International Fuel Gas Code 2015 International Fire Code Local Code of Ordinances or Unified Development Ordinance Zoning Code Food Code – Defer to State of Texas Food Code – (Ileen Heard, 2023) |

## 2.2 Permits (Define each as required)

| TBD | Permit | Submittal Requirements (Electronic/In-person, # Sets, Who Submits?) | Review Duration | Fees ($) | Stamp Sign | Payment Method |
| --- | --- | --- | --- | --- | --- | --- |
| ☐ | Planning / Zoning | Not applicable to this report. | Not applicable to this report. | Not applicable to this report. | Not applicable to this report. | Not applicable to this report. |
| ☐ | Building | Click or tap here to enter text. |  |  |  |  |
| ☐ | Mechanical | Click or tap here to enter text. |  |  |  |  |
| ☐ | Electrical | Click or tap here to enter text. |  |  |  |  |
| ☐ | Plumbing | Click or tap here to enter text. |  |  |  |  |
| ☐ | Structural | Click or tap here to enter text. |  |  |  |  |
| ☐ | Health | Click or tap here to enter text. |  |  |  |  |
|  | Special drawing requirements for Permit? | ☐ Yes ☐ No Click or tap here to enter text. <-Remove this text if nothing is written here. |  |  |  |  |
|  | Equipment cut sheet requirements. (Link to Sbux cut sheet manual) | ☐ Yes ☐ No Click or tap here to enter text. <-Remove this text if nothing is written here. |  |  |  |  |
|  | Metal Framework casework requirements (Adaptable Casework) - i.e. drawings, cut sheets, samples, videos, etc. | ☐ Yes ☐ No Click or tap here to enter text. <-Remove this text if nothing is written here. |  |  |  |  |
| ☐ | Fire Safety | Click or tap here to enter text. |  |  |  |  |
| ☐ | Other | Click or tap here to enter text. |  |  |  |  |
|  | Deferred Submittal |  |  |  |  |  |
| ☐ | Fire Protection | Click or tap here to enter text. |  |  |  |  |
| ☐ | Low Voltage | Click or tap here to enter text. |  |  |  |  |
| ☐ | Signage | Click or tap here to enter text. |  |  |  |  |
| ☐ | Other | Click or tap here to enter text. |  |  |  |  |

| TBD | Impact Fees | N/A | N/A |
| --- | --- | --- | --- |

## 2.3a Building

| TBD | Building Department ☐ Check if this section is NOT applicable. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ☐ | Requirements for Permit Submittal (Select all that apply) | ☐Cost of Construction ☐ GC license # ☐Sub Contractor list ☐GC Bonding ☐ Intake Fee ☐ Other describe |  |  |  |  |
| ☐ | What is the Total Duration for Building Plan Review | First Round Comments Days/weeks Second Round Review Days/weeks Plan Correction Days/weeks Total Review Days/weeks |  |  |  |  |
| ☐ | What are the procedures for re-issuing Plan Corrections? | Describe procedures/re-submittal requirements |  |  |  |  |
| ☐ | Construction must begin within how many days of permit issuance? | 180 days |  |  |  |  |
| ☐ | Is there a Permit Extension Available? | ☐ Yes ☐ No Duration If Yes, How do you request |  |  |  |  |
| ☐ | Is a separate demo permit required? | ☐ Yes ☐ No Requirements, Fees, Review duration |  |  |  |  |
| ☐ | Operating Hours (Permit Submittals) | Hours | Can a Demo permit be issued prior to bldg. permit release? |  | ☐ Yes ☐ No |  |
| ☐ | Is an appointment needed for submittal? | ☐Yes ☐ No If Yes, how far out are appointments? |  |  |  |  |
| ☐ | Does the jurisdiction have an expedited review process? | ☐Yes ☐ No If Yes, describe intake requirements |  | Is a GC required for permit submittal? |  | ☐Yes ☐ No |
| ☐ | What is the procedure for revising Approved Permit Drawings? | describe intake requirements, Fee, Review duration |  |  |  |  |
| ☐ | Is Landlord approval needed prior to Permit Submittal? | ☐Yes ☐ No If Yes, describe Requirements |  |  |  |  |
|  | Other submittal requirements | None stipulated. |  |  |  |  |

## 2.3b Planning / Zoning - NA

## 2.4 Additional Permit Information - NA

## 2.5 Fire Resistive Requirements - NA

Define the Separation Requirements between tenants at each of the following assemblies.

| TBD | Separation Requirements– The following is an interpretation of separation requirements only. This should be verified by Starbucks AOR. | ☐ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Exterior Wall | Type V-B construction, a 0-hour separation between exterior walls is required. (Per 2018 International Building Code Table 601.) |
| ☐ | Demising Wall | Separation requirements are dependent on the use group of the space. Separation between M or B and A-2 use groups require a fire separation of 1-hour (Sprinklered) and 2 hours (non-sprinklered). Separation between A and A occupancies require a 0-hour separation. Per 2018 International Building Code Table 508.4. If this is a standalone building, remove the wording above and add this instead: Not applicable to this report, no demising walls. |
| ☐ | Ceiling/Roof | Type V-B construction, a 0-hour separation between ceiling/roof is required. (Per 2018 International Building Code Table 601.) |
| ☐ | Floor | Type V-B construction, a 0-hour separation between floors is required. (Per 2018 International Building Code Table 601.) |

Define the Flame Spread Requirements of each of the following components.

|  | ## Flame Spread Requirements– The following is an interpretation of flame spread requirements only. This should be verified by Starbucks AOR. | ☐ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | ## Casework | Assuming A-2 occupancy, rooms and enclosed spaces can have class C finishes. Interior exit stairways, ramps, exit passageways, corridors, enclosures for exit access stairways and ramps require class B finishes. (Per 2018 International Building Code, Table 803.13) Starbucks uses Class C finish or better. Assuming B occupancy, rooms and enclosed spaces must have a Class C finish. Corridors and enclosures for exit access stairways and ramps must be Class C finish. Interior exit stairways and ramps and exit passageways must have a class B finish. (Per 2018 International Building Code, Table 803.13). Starbucks uses Class C finish or better. |
| ☐ | ## Substrate | Assuming A-2 occupancy, rooms and enclosed spaces can have class C finishes. Interior exit stairways, ramps, exit passageways, corridors, enclosures for exit access stairways and ramps require class B finishes. (Per 2018 International Building Code, Table 803.13) Starbucks uses Class C finish or better. Assuming B occupancy, rooms and enclosed spaces must have a Class C finish. Corridors and enclosures for exit access stairways and ramps must be Class C finish. Interior exit stairways and ramps and exit passageways must have a class B finish. (Per 2018 International Building Code, Table 803.13). Starbucks uses Class C finish or better. |
| ☐ | ## Sealants | Assuming A-2 occupancy, rooms and enclosed spaces can have class C finishes. Interior exit stairways, ramps, exit passageways, corridors, enclosures for exit access stairways and ramps require class B finishes. (Per 2018 International Building Code, Table 803.13) Starbucks uses Class C finish or better. Assuming B occupancy, rooms and enclosed spaces must have a Class C finish. Corridors and enclosures for exit access stairways and ramps must be Class C finish. Interior exit stairways and ramps and exit passageways must have a class B finish. (Per 2018 International Building Code, Table 803.13). Starbucks uses Class C finish or better. |
| ☐ | ## Murals | Assuming A-2 occupancy, rooms and enclosed spaces can have class C finishes. Interior exit stairways, ramps, exit passageways, corridors, enclosures for exit access stairways and ramps require class B finishes. (Per 2018 International Building Code, Table 803.13) Starbucks uses Class C finish or better. Assuming B occupancy, rooms and enclosed spaces must have a Class C finish. Corridors and enclosures for exit access stairways and ramps must be Class C finish. Interior exit stairways and ramps and exit passageways must have a class B finish. (Per 2018 International Building Code, Table 803.13). Starbucks uses Class C finish or better. |
| ☐ | ## Furniture | Assuming A-2 occupancy, rooms and enclosed spaces can have class C finishes. Interior exit stairways, ramps, exit passageways, corridors, enclosures for exit access stairways and ramps require class B finishes. (Per 2018 International Building Code, Table 803.13) Starbucks uses Class C finish or better. Assuming B occupancy, rooms and enclosed spaces must have a Class C finish. Corridors and enclosures for exit access stairways and ramps must be Class C finish. Interior exit stairways and ramps and exit passageways must have a class B finish. (Per 2018 International Building Code, Table 803.13). Starbucks uses Class C finish or better. |
| ☐ | ## Fabric | Assuming A-2 occupancy, rooms and enclosed spaces can have class C finishes. Interior exit stairways, ramps, exit passageways, corridors, enclosures for exit access stairways and ramps require class B finishes. (Per 2018 International Building Code, Table 803.13) Starbucks uses Class C finish or better. Assuming B occupancy, rooms and enclosed spaces must have a Class C finish. Corridors and enclosures for exit access stairways and ramps must be Class C finish. Interior exit stairways and ramps and exit passageways must have a class B finish. (Per 2018 International Building Code, Table 803.13). Starbucks uses Class C finish or better. |
|  | ## Other | None stipulated. |

## 2.6 Egress

| TBD | Egress (Existing Buildings) |  |  | ☐ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- | --- | --- |
|  | Describe the quantity and configuration of existing exits |  |  |  |  |
|  |  | Exit 1 | Exit 2 |  | Exit 3 |
| ☐ | Location Size Hardware Accessible Actuator | Not applicable to this report. Due diligence report only. | Not applicable to this report. Due diligence report only. |  | Not applicable to this report. Due diligence report only. |
| ☐ | Describe any path of travel restrictions based on the current egress layout |  |  | Not applicable to this report. Due diligence report only. |  |
| ☐ | Are existing exits permissible under current building code? |  |  | Not applicable to this report. Due diligence report only. |  |
| ☐ | Are exits through adjoining areas (work/storage) permitted? |  |  | ☐Yes ☒ No |  |
| ☐ | Is there an accessible path of Travel to a public way? |  |  | Not applicable to this report. Due diligence report only. |  |
| ☐ | Are there stairs, ramps, or elevators, included as part of the egress path |  |  | Not applicable to this report. Due diligence report only. |  |
| ☐ | Is signage and emergency lighting installed as required per Code |  |  | Not applicable to this report. Due diligence report only. |  |

## 2.7 Restroom Requirements

| TBD | Restrooms (Existing) | ☐ Check if this section is NOT applicable. |  |  |
| --- | --- | --- | --- | --- |
| ☐ | How many restrooms are required for Starbucks customer and partner use? | Per 2021 International Plumbing Code Table 403.1 For Assembly A-2 occupancy: 1 toilet per 75 male occupants 1 toilet per 75 female occupants For B occupancy: 1 toilet per 25 occupants up to 50 occupants, and 1 toilet per 50 occupants for anything over 50. Per 2021 International Building Code, Table 1004.5: Assembly use has an occupancy load of 15 SF Net per person. B use has an occupancy load of 150 gross per person. Calculation for A-2: __ sf/15 sf net per person =____ occupants / 2 restrooms per gender (male/female) = ____ occupants per sex If considered A-2 occupancy, one restroom is required per gender, two total. Calculation for B: ____ sf/150 sf gross = ____ occupants For B occupancy, one gender neutral restroom is required since below 25 occupants. A breakdown of square footage has not been provided; therefore, an accurate calculation cannot be provided. |  |  |
| ☐ | How many restrooms are provided for Starbucks Partner use? | Not applicable to this report. Due diligence report only. |  |  |
| ☐ | Are the existing restrooms handicap accessible? | Not applicable to this report. Due diligence report only. |  |  |
| ☐ | Are common restrooms available onsite? | No common restrooms. |  |  |
| ☐ | What is the distance path of travel to common restrooms? | No common restrooms. | Hours of Operation | Not applicable to this report. |
| ☐ | Are Unisex Restrooms provided? | Not applicable to this report. Due diligence report only. | Are they allowed? | ☒Yes ☐ No |
| ☐ | Does the building have a water use reduction plan? | Not applicable to this report. Due diligence report only. |  |  |

## 2.8 Health Code Requirements

| TBD | Health Code | ☐ Check if this section is NOT applicable. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ | Is a menu required as part of the permit Submittal? | ☒Yes ☐ No |  |  |  |
| ☐ | Is a 3-compartment sink required? | ☒Yes ☐ No 4-301.12 Manual Warewashing, Sink Compartment Requirements. (A) Except as specified in ¶ (C) of this section, a sink with at least 3 compartments shall be provided for manually washing, rinsing, and SANITIZING EQUIPMENT and UTENSILS. Pf (B) Sink compartments shall be large enough to accommodate immersion of the largest EQUIPMENT and UTENSILS. If EQUIPMENT or UTENSILS are too large for the WAREWASHING sink, a WAREWASHING machine or alternative EQUIPMENT as specified in ¶ (C) of this section shall be used. |  | What size drain board is required | 4-301.13 Drainboards. Drainboards, utensil racks, or tables large enough to accommodate all soiled and cleaned items that may accumulate during hours of operation shall be provided for necessary utensil holding before cleaning and after sanitizing. (2017 FDA Food Code). |
| ☐ | Is a hot water Sanitizing Dishwasher required? | ☐Yes ☒ No |  |  |  |
| ☐ | Is a garbage disposal required? | ☐Yes ☒ No |  |  |  |
| ☐ | Are kitchen equipment cut sheets required at permit submission? | ☒Yes ☐ No |  |  |  |
| ☐ | Are special ceiling/wall/floor/casework finishes required? | Cafe Area / FOH: ☒Yes ☐ No 6-201.11 Floors, Walls, and Ceilings. Except as specified under § 6-201.14 and except for antislip floor coverings or applications that may be used for safety reasons, floors, floor coverings, walls, wall coverings, and ceilings shall be designed, constructed, and installed so they are SMOOTH and EASILY CLEANABLE. |  |  | Prep Area / BOH: ☒Yes ☐ No 6-201.11 Floors, Walls, and Ceilings. Except as specified under § 6-201.14 and except for antislip floor coverings or applications that may be used for safety reasons, floors, floor coverings, walls, wall coverings, and ceilings shall be designed, constructed, and installed so they are SMOOTH and EASILY CLEANABLE. |
| ☐ | Are special Lighting fixtures required above food prep areas? | ☒Yes ☐ No 6-202.11 Light Bulbs, Protective Shielding. (A) Except as specified in (B) of this section, light bulbs shall be shielded, coated, or otherwise shatter-resistant in areas where there is exposed food; clean equipment, utensils, and linens; or unwrapped single-service and single-use articles. (B) Shielded, coated, or otherwise shatter-resistant bulbs need not be used in areas used only for storing food in unopened packages, if: (1) The integrity of the packages cannot be affected by broken glass falling onto them; and (2) The packages are capable of being cleaned of debris from broken bulbs before the packages are opened. (C) An infrared or other heat lamp shall be protected against breakage by a shield surrounding and extending beyond the bulb so that only the face of the bulb is exposed. (2017 FDA Food Code). |  |  |  |
| ☐ | Are sneeze guards required? | ☒Yes ☐ No 3-306.11 Food Display. Except for nuts in the shell and whole, raw fruits and vegetables that are intended for hulling, peeling, or washing by the consumer before consumption, food on display shall be protected from contamination by the use of packaging; counter, service line, or salad bar food guards; display cases; or other effective means. (2017 FDA Food Code). |  |  |  |
| ☐ | What are the local hand sinks requirements? | 5-203.11 Handwashing Sinks. (A) Except as specified in ¶¶ (B) and (C) of this section, at least 1 HANDWASHING SINK, a number of HANDWASHING SINKS necessary for their convenient use by EMPLOYEES in areas specified under § 5-204.11, and not fewer than the number of HANDWASHING SINKS required by LAW shall be provided. (B) If APPROVED and capable of removing the types of soils encountered in the FOOD operations involved, automatic handwashing facilities may be substituted for HANDWASHING SINKS in a FOOD ESTABLISHMENT that has at least 1 HANDWASHING SINK. (C) If APPROVED, when FOOD exposure is limited and HANDWASHING SINKS are not conveniently available, such as in some mobile or TEMPORARY FOOD ESTABLISHMENTS or at some VENDING MACHINE LOCATIONS, EMPLOYEES may use chemically treated towelettes for handwashing. (2017 FDA Food Code). |  |  |  |
| ☐ | Are there remote storage Requirements? | ☒Yes ☐ No 3-305.11 Food Storage. (A) Except as specified in (B) and (C) of this section, food shall be protected from contamination by storing the food: In a clean, dry location; Where it is not exposed to splash, dust, or other contamination; and At least 15 cm (6 inches) above the floor. (B) Food in packages and working containers may be stored less than 15 cm (6 inches) above the floor on case lot handling equipment as specified under 4-204.122. (C) Pressurized beverage containers, cased food in waterproof containers such as bottles or cans, and milk containers in plastic crates may be stored on a floor that is clean and not exposed to floor moisture. 3-305.12 Food Storage, Prohibited Areas. Food may not be stored: In locker rooms; In toilet rooms; In dressing rooms; In garbage rooms; In mechanical rooms; Under sewer lines that are not shielded to intercept potential drips; Under leaking water lines, including leaking automatic fire sprinkler heads, or under lines on which water has condensed; Under open stairwells; or Under other sources of contamination. (2017 FDA Food Code). |  |  |  |
| ☐ | Does the AHJ have an approved list of plumbing fixtures? | 401.2 Prohibited fixtures and connections. Water closets having a concealed trap seal or an unventilated space or having walls that are not thoroughly washed at each discharge in accordance with ASME 112.19.2/CSA b45.1 shall be prohibited. Any water closet that permits siphonage of the contents of the bowl back into the tank shall be prohibited. Trough urinals shall be prohibited. 402.1 Quality of fixtures Plumbing fixtures shall be constructed of approved materials, with smooth, impervious surfaces, free from defects and concealed fouling surfaces, and shall conform to standards cited in this code. All porcelain enameled surfaces on plumbing fixtures shall be acid resistant. 402. Materials for specialty fixtures Materials for specialty fixtures not otherwise covered in this code shall be of stainless steel, soapstone, chemical stoneware or plastic, or shall be lined with lead, copper-based alloy, nickel-copper alloy, corrosion-resistant steel or other material especially suited to the application for which the fixture is intended. (2021 IPC) |  |  |  |
| ☐ | Are we required to install a grease interceptor? | ☒Yes ☐ No |  |  |  |
|  | If required to install a GGI yes is there a process for submitting a variance? | Describe variance process and timeline for approval |  |  |  |
|  | Casework |  | ☐ Is this an “Adaptable Casework Project” |  |  |
| ☐ | Will this store require a casework pre submittal or special attention to ensure approval of Starbucks casework system? (Adaptable Casework only) | ☐Yes ☒ No If yes please identify in Link to jurisdictional tracker |  |  |  |
| ☐ | Is there an online health Code | ☒Yes ☐ No https://www.fda.gov/media/110822/download |  |  |  |
| ☐ | Are there special material termination requirements? | Cafe Area / FOH: ☒Yes ☐ No Prep Area / BOH: ☒Yes ☐ No |  |  |  |
|  |  | 6-201.13 Floor and Wall Junctures, Coved, and Enclosed or Sealed. (A) In food establishments in which cleaning methods other than water flushing are used for cleaning floors, the floor and wall junctures shall be coved and closed to no larger than 1 mm (one thirty-second inch). (B) The floors in food establishments in which water flush cleaning methods are used shall be provided with drains and be graded to drain, and the floor and wall junctures shall be coved and sealed. (2017 FDA Food Code). |  |  |  |
| ☐ | Case work Base Requirement | ☐Box Base ☒ 6-inch Legs |  |  |  |
| ☐ | Is there a LRV (Light reflectance value) requirement for cabinets | ☐Yes ☐ No If yes what is the LRV value |  |  |  |
| ☐ | What are the requirements for permit Submittal? | Describe requirements |  |  |  |
| ☐ | Describe any other special requirements. | Describe |  |  |  |

## 2.9 General Accessibility Requirements

| TBD | General Accessibility | ☐ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Is the Front Counter accessible? | Not applicable to this report. Due diligence report only. |
| ☐ | Identify the path of travel from accessible parking to accessible store entrance | Not applicable to this report. Due diligence report only. |
| ☐ | What is the handicapped accessible seating requirement? | Fixed seating – 4-25 spaces require 1 wheelchair space, 26-50 requires 2 wheelchair spaces. At least one companion seat is required for each wheelchair space provided. At least 5% but not less than 1 of the total number of aisle seating shall be designated aisle seating and shall be the aisle seats located closest to the accessible route. (2018 International Building Code Chapter 11) |
| ☐ | Is outdoor seating accessible | Not applicable to this report. Due diligence report only. |
| ☐ | Are there any existing/proposed grade changes that require ramps? | Not applicable to this report. Due diligence report only. |
| ☐ | Is a lift an acceptable alternative to a ramp? | ☒Yes ☐ No |
|  | Describe any special requirements | None stipulated. |

## 2.10 Seating

| TBD | Indoor Seating | ☐ Check if this section is NOT applicable. |  |  |
| --- | --- | --- | --- | --- |
| ☐ | Is there existing seating? | Not applicable to this report. Due diligence report only. | How many seats | Not applicable to this report. Due diligence report only. |
| ☐ | What types and style? | Not applicable to this report. Due diligence report only. |  |  |
| ☐ | Are the number of seats restricted? | ☒Yes ☐ No Per 2018 International Building Code, Table 1004.5: Assembly with fixed seats unconcentrated (tables and chairs) has an occupancy load of 15 sf net. |  |  |
| ☐ | Are stand-up counters to be included in the seating calculations? | ☒Yes ☐ No |  |  |

| TBD | Outdoor Seating | ☐ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Is outdoor seating allowed? | ☒Yes ☐ No |
| ☐ | Is a permit required for outdoor seating? | ☐Yes ☒ No If Yes, list requirements |
| ☐ | Maximum seating area permitted | sqft/m2 |
| ☐ | Maximum number of seats permitted | Per 2018 International Building Code, Table 1004.5: Assembly with fixed seats unconcentrated (tables and chairs) has an occupancy load of 15 sf net. B use has an occupancy load of 150 gross per person. Add occupancy load if on plans There was no breakdown of square footage provided. If using the site plans that listed ____ square footage for the patio, to give an approximate calculation: Seating for A-2 would be ____ and seating for B would be ____. This does not mean that ____ or ____ seats are required, but available to be placed in the space. If no square footage was provided at all, use this verbiage: A breakdown of square footage has not been provided; therefore, an accurate seating calculation cannot be completed. |
| ☐ | Is an enclosure required? | ☐Yes ☐ No If Yes, Describe. |
| ☐ | Is the outdoor seating area calculated as part of the restroom calculation? | ☒Yes ☐ No |
| ☐ | Does the L.L have required specifications? | ☐Yes ☐ No Up to landlord. |
| ☐ | Does the landlord have furniture restrictions or requirements (including umbrellas or furniture storage)? | ☐Yes ☐ No Up to landlord. |
| ☐ | Is parking available for expanded seating | ☐Yes ☒ No |

# 3. ZONING & SITE INFORMATION

## 3.1 General Evaluation – N/A

## 3.2 Easements – N/A

## 3.3 Land Division – N/A

## 3.4 General Zoning and Use Requirements

| TBD | Zoning and Use Requirements | ☐ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Property Zone |  |  |

## 3.5 Environmental – N/A

## 3.6 Storm Water – N/A

## 3.7 Site Signage Requirements

| TBD | Signage - External Signs |  |  | ☐ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- | --- | --- |
| ☐ | Are Illuminated Sign permissible? |  |  | ☒Yes ☐ No |  |
| ☐ | Are there any signage restrictions? |  |  | ☒Yes ☐ No |  |
|  | Elevation | Is there existing signage? | If Yes, identify type of signage. |  | Signage Allowance per Code? |
| ☐ | North | ☐Yes ☐ No ☒ NA | Not applicable to this report. Due diligence report only. |  | Describe |
| ☐ | South | ☐Yes ☐ No ☒ NA | Not applicable to this report. Due diligence report only. |  | Describe |
| ☐ | East | ☐Yes ☐ No ☒ NA | Not applicable to this report. Due diligence report only. |  | Describe |
| ☐ | West | ☐Yes ☐ No ☒ NA | Not applicable to this report. Due diligence report only. |  | Describe |
| ☐ | What is the maximum number of signs allowed by type? |  |  | Describe quantity and type |  |
| ☐ | What is the maximum area/size of building signage allowed? |  |  | sqft/m2 |  |
| ☐ | Is there a height of sign restriction in place and what are the location parameters (height, projection, clearance, etc.)? |  |  | ☐Yes ☐ No If Yes, Describe. |  |

| TBD | Awning | ☐ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Are there existing awnings? | Not applicable to this report. Due diligence report only. |
| ☐ | What is the maximum number of awnings allowed? | # |
| ☐ | What is the maximum area of awnings, clearance from grade, projection from building allowed? | Max. area: sqft/m2 Min. clearance: ft/m Max. projection: ft/m |
| ☐ | Are Logos and Lettering allowed on the awnings? | ☐Yes ☐ No If Yes, do they count towards total allowed signage area |
| ☐ | Will awnings (without graphics) require a sign permit? | ☐Yes ☐ No If Yes, note in Permits |

## 3.8 Building Setbacks and Height Restrictions – N/A

## 3.9 Landscaping – N/A

## 3.10 Parking

| TBD | Parking | ☐ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Is there existing parking? | Not applicable to this report. Due diligence report only. |
| ☐ | What is the formula for calculating the number of required parking stalls? | Describe |
| ☐ | Will a variance be required to achieve parking count based on the new plan? | ☐Yes ☒ No |

## 3.11 Drive-Thru Site Requirements

| TBD | Drive-Thru Site Requirements | ☐ Check if this section is NOT applicable. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ | Is there an existing drive thru onsite? | Not applicable to this report. Due diligence report only. |  |  |  |
| ☐ | Are there local queuing requirements for the Drive-Thru lane? | ☐Yes ☒ No If Yes, define the formula for calculation |  |  |  |
| ☐ | Does the Drive Thru (DT) accommodate a 4 to 5 car stack? | ☒Yes ☐ No (at least 80ft/2438cm from the center of the DT window to speaker post) |  |  |  |
| ☐ | Existing Drive-Thru (DT): what is the drive aisle turn radius. | Not applicable to this report. Due diligence report only. |  |  |  |
| ☐ | Existing Drive-Thru (DT): Are there sufficient visual warnings and physical separations at crosswalks in vehicular lanes? | Not applicable to this report. Due diligence report only. |  | height of the canopy? | Not applicable to this report. Due diligence report only. |
| ☐ | Are there any restrictions on the menu board installation? |  | ☐Yes ☐ No If Yes, Describe. |  |  |  |
| ☐ | Is there an existing preview board? | Not applicable to this report. Due diligence report only. |  |  |  |
| ☐ | What material is drive through pavement? Note: DT lanes other than asphalt or concrete pavement will require alternate solutions for sensor loops | Not applicable to this report. Due diligence report only. |  |  |  |

## 3.12 Deliveries – N/A

3.13 Site Amenities

| TBD | Site Amenities | ☒ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Is there an outdoor patio? | Not applicable to this report. Due diligence report only. |
| ☐ | Are there physical separations for patios, sidewalks, etc. from vehicular areas? | Not applicable to this report. Due diligence report only. |

# 4. UTILITIES

## 4.1 HVAC

| TBD | General HVAC Information | ☐ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Is there an existing HVAC system? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Who maintains existing system? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is the Tenant system part of a larger central system? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Type of System (Split, CAV, VAV) |  | Not applicable to this report. Due diligence report only. |
| ☐ | Age/Condition of System |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is a balancing report available? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Are there equipment screening requirements? |  | ☒Yes ☐ No 1510.6 Mechanical Equipment Screens. Mechanical equipment screens shall be constructed of the materials specified for the exterior walls in accordance with the type of construction of the building. Where the fire separation distance is greater than 5 feet (1524 mm), mechanical equipment screens shall not be required to comply with the fire-resistance rating requirements. 1510.6.1 Height Limitations. Mechanical equipment screens shall not exceed 18 feet (5486 mm) in height above the roof deck, as measured to the highest point on the mechanical equipment screen. Exception: Where located on buildings of Type IA construction, the height of mechanical equipment screens shall not be limited. 1510.6.2 Type I, II, III or IV Construction. Regardless of the requirements in Section 1510.6, mechanical equipment screens that are located on the roof decks of buildings of Type I, II, III or IV construction shall be permitted to be constructed of combustible materials in accordance with any one of the following limitations: The fire separation distance shall be not less than 20 feet (6096 mm) and the height of the mechanical equipment screen above the roof deck shall not exceed 4 feet (1219 mm) as measured to the highest point on the mechanical equipment screen. The fire separation distance shall be not less than 20 feet (6096 mm) and the mechanical equipment screen shall be constructed of fire-retardant-treated wood complying with Section 2303.2 for exterior installation. Where exterior wall covering panels are used, the panels shall have a flame spread index of 25 or less when tested in the minimum and maximum thicknesses intended for use, with each face tested independently in accordance with ASTM E84 or UL 723. The panels shall be tested in the minimum and maximum thicknesses intended for use in accordance with, and shall comply with the acceptance criteria of, NFPA 285 and shall be installed as tested. Where the panels are tested as part of an exterior wall assembly in accordance with NFPA 285, the panels shall be installed on the face of the mechanical equipment screen supporting structure in the same manner as they were installed on the tested exterior wall assembly. (2018 International Building Code) Local Code requirements to replace above if applicable |
| ☐ | Main supply duct or pipe size |  | Not applicable to this report. Due diligence report only. |
| ☐ | CFM available/static pressure |  | Not applicable to this report. Due diligence report only. |
| ☐ | Total CFM outdoor air available |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is there a plenum |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is replacement, relocations or upgrade recommended? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is noise abatement required? |  | ☐Yes ☐ No |

| TBD | Cooling Unit | ☒ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Is there an existing system |  |  |
| ☐ | Type of system |  |  |
| ☐ | Location |  |  |
| ☐ | Manufacturer/Model No |  |  |
| ☐ | Physical size (WxHxD) |  |  |
| ☐ | Distance from Tenant space |  |  |

| TBD | Existing Unit “A” |  |  |  | ☒ Check if this section is NOT applicable. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | Type (fan coil, VAV, etc.)) |  |  |  |  |  |  |  |  |  |
| ☐ | Type (packaged rooftop, split system, heat pump) |  |  |  |  |  |  |  |  |  |
| ☐ | Manufacturer/Model No |  |  |  |  |  |  |  |  |  |
| ☐ | Physical size (WxHxD) |  |  |  |  |  |  |  |  |  |
| ☒ | Voltage |  | Amps |  |  |  | Phase |  | Source |  |
| ☒ | Heating capacity |  |  |  |  |  |  | Cooling capacity |  |  |
| ☐ | Energy Star |  |  |  |  |  |  |  |  |  |
| ☐ | SEER/EER/IPLV |  |  |  |  |  |  |  |  |  |
| ☐ | Economizer |  |  |  |  |  |  |  |  |  |
| ☐ | Refrigerant Type |  |  |  |  |  |  |  |  |  |
| ☐ | Age/Condition of units |  |  |  |  |  |  |  |  |  |
| ☐ | Control/Thermostat |  |  |  |  |  |  |  |  |  |
| ☐ | Is replacement or upgrade recommended? |  |  |  |  |  |  |  |  |  |

*Note: If more than one unit exists, add additional tables and describe

| TBD | Fresh Air Intake (Locate on drawings) | ☒ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Size of louver |  |  |
| ☐ | Proximity of intake to: |  |  |
| ☐ | Is the site open to an enclosed area that allows smoking? |  |  |

| TBD | Restroom Exhaust (Locate on drawings) | ☐ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Is the R.R. exhaust part of a larger central system? |  |  |
| ☐ | Existing fan type |  |  |
| ☐ | Fan location |  |  |
| ☐ | Sconce rating |  |  |
| ☐ | Manufacturer/Model No. |  |  |
| ☐ | Exhaust Volume |  |  |
| ☐ | Where does exhaust terminate? |  |  |
| ☐ | Is current installation code compliant? |  |  |

| TBD | Water Heater | ☒ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Is there an existing water heater in the space? |  |  |
| ☐ | Is there an existing hot water heater |  |  |
| ☐ | Manufacturer/Model No. |  |  |
| ☐ | Size |  |  |
| ☐ | Age |  |  |
| ☐ | Source of power |  |  |
| ☐ | kW/h |  |  |
| ☐ | Heating Capacity |  |  |
| ☐ | Exhaust vent |  |  |
| ☐ | Locate Exhaust vent termination |  |  |
| ☐ | Does existing heater need to be relocated / replaced? |  |  |
| ☐ | Does it service other tenants? |  |  |

## 4.2 Electrical Service

| TBD | Electrical Service (Existing) | ☒ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Is a 120-208V, 3-phase, 4 wire, and 400 amp service available? (This is the min. service Req.) |  |  |
| ☐ | Location of existing primary service line? |  |  |
| ☐ | Where are the meter and shut off located? |  |  |
| ☐ | Are the electrical panels provided within Starbucks space or remoted |  |  |

|  | Electrical Service (New) | ☒ Check if this section is NOT applicable |
| --- | --- | --- |

| TBD | Electrical Service (Upgrade) |  | ☒ Check if this section is NOT applicable. |
| --- | --- | --- | --- |
| ☐ | Estimate the time required for a service upgrade |  |  |
| ☐ | Who is responsible for the upgrade cost? |  |  |
| ☐ | Is a transformer required? |  |  |
| ☐ | What is the total amount of power available to the Bldg.? |  |  |
| ☐ | Are there any code violations that need correction? |  |  |

| TBD | Electrical Bldg. | ☒ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Existing meter number |  |  |
| ☐ | Does the meter supply Starbucks site only? |  |  |
| ☐ | Dist. from meter to Panel |  |  |
| ☐ | Dist. from Panel to Transformer |  |  |
| ☐ | Transformer Mounting Type |  |  |
| ☐ | Transformer - Primary |  |  |
| ☐ | Transformer - Secondary |  |  |
| ☐ | Existing Electrical Panel |  |  |
| ☐ | Recommendations for service and panel if relocation necessary |  |  |
| ☐ | Is there sufficient clearance/ separation around electrical equipment? |  |  |
| ☐ | Describe the HVAC disconnects |  |  |

## 4.3 Water Service

Note:  1 1/2” (25.4mm) line 50-60 PSI standard.

| TBD | Water Service | ☒ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Is this service shared with other tenants? |  |  |
| ☐ | Size of existing main and location Does the main need to be upgraded |  |  |
| ☐ | Water meter number |  |  |
| ☐ | What is the dynamic water pressure? |  |  |

## 4.4 Water Test

Note:  Refer to the Instructions document regarding the process for obtaining a test kit prior to site visit.

| TBD | Water Test | ☒ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Was a water sample gathered and submitted per Instructions? |  |  |
| ☐ | What is the water temperature? |  |  |

## 4.5 Sanitary Sewer Service

| TBD | Sanitary Sewer Service | ☐ Check if this section is NOT applicable. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ | Is sanitary sewer available? |  |  | Not applicable to this report. Due diligence report only. |  |
| ☐ | What is the existing service size? |  |  | Not applicable to this report. Due diligence report only. |  |
| ☐ | Is there an existing septic tank? |  |  | Not applicable to this report. Due diligence report only. |  |
| ☐ | Is Scoping of Line Recommended? |  |  | ☒Yes ☐ No It is always recommended to scope the line. |  |
| Grease Interceptors - For this section please review Starbucks FOG Guidelines located on the SDRC. Link - For FOG Due Diligence on Company Owned and License Stores Starbucks has partnered with Schier Products. - The jurisdictional requirements for your project have been uploaded to the attached spread sheet for review. Link - Add the appropriate information to the section below and attach the preapproved calculations. |  |  |  |  |  |
| ☐ | Is a Grease Interceptor required? |  | ☐Yes ☐ No |  |  |
| ☐ | If an Interceptor is required Describe: |  | Type: GGI / HGI Dimensions: Dimensions Size: Liquid Capacity Requirement- Gallons/Liters Flow Rate: Flow Rate Requirements |  |  |
| ☐ | What accessories are required? |  | List Accessories: Riser, Remote pump-out, Anchor Kits, Support Kits, Clamping Collars, etc. |  |  |
| ☐ | What Fixtures are required to be plumbed through the interceptor? |  | List Fixtures: 3 comp, Mop Sink, Floor Sinks, Rinse Sinks, Dishwasher, etc. |  |  |
| ☐ | Is there an existing grease interceptor? |  | Not applicable to this report. Due diligence report only. |  |  |
| ☐ | If there’s an existing interceptor Describe: |  | Not applicable to this report. Due diligence report only. |  |  |
| ☐ | What existing fixtures are plumbed through the interceptor? |  | Not applicable to this report. Due diligence report only. |  |  |
| ☐ | GI Servicing Requirements |  | Who is responsible for servicing? |  | Frequency of Service? |
| ☐ | What are requirements for FOG submittal? |  | describe plan requirements for submittal |  |  |
| ☐ | What are the requirements for a Variance Request? |  | Describe variance request requirements here Describe process and schedule here |  |  |

| Department | Name, Title | Address, City, State, Zip, County | Phone, Fax, E-mail, website |
| --- | --- | --- | --- |
| Sewer |  |  | Ph: Email: Website: |

Please attach the Grease Interceptor Calculation Report for your project in the space provided below.

You can download the specific report for your project following the attached link.  Link

Starbucks Grease Interceptor Calculation

## 4.6 Cable Service

| TBD | Telephone / Data Service | ☐ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
|  | Please leverage the FCC’s free Broadband mapping tools to help assess connectivity speed and prospective providers for your site. For more information please reference Site Internet & Voice Systems SDRC Page and either the FCC Broadband Map or the Canada Broadband Map |  |  |
| ☐ | Is there existing service on site? |  | ☐Yes ☒ No |
| If NO |  |  |  |
| ☐ | Locate closest off-site cable/fiber drop. |  |  |
| ☐ | What types of service is available? Note: Starbucks Standard is 50x10 Fiber optic or Cable [DSL in not acceptable.] |  |  |
| ☐ | Has the owner engaged with a local telco company to determine connectivity to the site? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is the owner willing to work with Tenant service providers to ensure connectivity? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Are there any restriction to the site which would limit telco connectivity? |  | Not applicable to this report. Due diligence report only. |
| If Yes |  |  |  |
| ☐ | Are location of existing phone and internet service lines (overhead or underground) |  | Not applicable to this report. Due diligence report only. |
| ☐ | What type of service is provided |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is there an exist. Telco pedestal or vault on site? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Has the owner restricted access to any internet providers? Does the owner have an exclusive agreement with any internet provider? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is there an established telephone demarcation point in the Tenants Space? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is there power within 10’ of the demarc? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is there wireless cellular signal (Verizon or AT&T) |  | Not applicable to this report. Due diligence report only. |
| ☐ | Are there Minimum (2) conduits in place for internet and telephone services? |  | Not applicable to this report. Due diligence report only. |

## 4.7 Gas Service

| TBD | Gas Service | ☒ Check if this section is NOT applicable. Not applicable to this report. Due diligence report only. |  |
| --- | --- | --- | --- |
| ☐ | Is natural gas available on site? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is there an (e) meter on-site |  | Not applicable to this report. Due diligence report only. |
| ☐ | Location of existing stub |  | Not applicable to this report. Due diligence report only. |
| ☐ | Size of existing gas line |  | Not applicable to this report. Due diligence report only. |

## 4.8 Solid Waste and Recycling Services

| TBD | Solid Waste |  | ☐ Check if this section is NOT applicable. |
| --- | --- | --- | --- |
| ☐ | Are there solid waste services? | Not applicable to this report. Due diligence report only. |  |
| ☐ | Is a dumpster provided? | Not applicable to this report. Due diligence report only. |  |
| ☐ | Is a dumpster enclosure provided? | Not applicable to this report. Due diligence report only. |  |
| ☐ | Are there drainage requirements? | ☒Yes ☐ No |  |
| ☐ | Is an internal trash room provided? | Not applicable to this report. Due diligence report only. |  |

| TBD | Recycling Service | ☐ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Are there existing recycling services? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Is a recycle bin provided? |  | Not applicable to this report. Due diligence report only. |
| ☐ | Does the building have an existing internal recyclable sorting area? |  | Not applicable to this report. Due diligence report only. |
| ☐ | What waste streams are available to be recycled? |  | Describe materials - glass, cardboard, etc |

## 4.9 Fire Sprinkler

| TBD | Fire Requirements (Site) | ☐ Check if this section is NOT applicable. |
| --- | --- | --- |
|  | Fire Requirements (Building) |  |
| ☐ | Is there (e) sprinkler service to our space? | Not applicable to this report. Due diligence report only. |
| ☐ | Does the building require sprinklers? | ☐ Yes ☐ No A breakdown of square footage has not been provided; therefore, an accurate calculation cannot be completed. If the occupant load is below 50 then the space can be considered B occupancy. If the occupant load is above 50 then the space can be considered A-2 occupancy. If the occupancy load is below 100 sprinklers will not be required. 903.2.1.2 2018 International Building Code It is likely that the occupancy load will be less than 100 once the final tenant fit out is complete and the occupancy load calculation is completed by the AOR. |
| ☐ | Will the (e) system need to be modified? | Dependent on design |
| ☐ | Describe permitting for sprinkler system? | ☐w/ Building Submittal ☐ Deferred |
| ☐ | Is an annunciator required? | ☐Yes ☒ No 13.7.2.20.3 Annunciator Panel An annunciator panel, connected to the fire alarm system, shall be provided at a location readily accessible from the primary point of entry for emergency response personnel. [101: 32.3.3.4.3] (Per 2018 International Fire Code) |
|  | Fire Requirements (Alarm) |  |
| ☐ | Is a fire alarm panel required? | ☐Yes ☒ No A breakdown of square footage has not been provided; therefore, an accurate calculation cannot be completed. Fire alarm systems shall be installed in Group A occupancies where the occupant load due to the assembly occupancy is 300 or more. Fire alarm systems shall be installed in Group B occupancies where the occupant load of all floors is 500 or more, occupant load is more than 100 persons above or below the lowest level of exit discharge, or fire area contains an ambulatory care facility. (Per 2018 International Building Code) The occupancy load will be less than 300 once the final tenant fit out is complete and the occupancy load calculation is completed by the AOR. |
| ☐ | Is there an existing fire alarm system? | Not applicable to this report. Due diligence report only. |

## 4.10 Other Requirements

| TBD | Other Requirements |  | ☐ Check if this section is NOT applicable. |
| --- | --- | --- | --- |
| ☐ | Do any MEP utilities requiring access through adjacent tenant space? | Not applicable to this report. Due diligence report only. |  |
|  | Other | None stipulated. |  |

# 5. BUILDING INFORMATION

## 5.1 General information

| TBD | General Evaluation | ☒ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Interior Condition |  |
| ☐ | Exterior Condition |  |
| ☐ | Are there tenants above / below space? |  |
| ☐ | Tenants adjacent to space? |  |
| ☐ | Is building LEED Certified? (LEED) |  |
| ☐ | Building hours of operation |  |

## 5.2 Storefront & Windows

| TBD | Storefront & Windows * | ☒ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Assemblies |  |
| ☐ | Location |  |
| ☐ | Finishes / Color |  |
| ☐ | Single or double glazed |  |
| ☐ | Tinted glaze |  |
| ☐ | Awning / Shading |  |
| ☐ | Vestibule |  |

* Note: If more than one unit exists, add additional tables as appropriate

5.3 Exterior Doors and Frames

| TBD | Exterior Doors and Frames * | ☒ Check if this section is NOT applicable. |  |  |
| --- | --- | --- | --- | --- |
| ☐ | Assemblies |  | Fire Rated |  |
| ☐ | Single or double glazed |  |  |  |
| ☐ | Condition |  |  |  |
| ☐ | Threshold Condition |  |  |  |
| ☐ | Existing Hardware |  |  |  |
| ☐ | Are doors ADA compliant? |  |  |  |
| ☐ | Awning / Shading |  |  |  |

* Note: If more than one unit exists, add additional tables as appropriate

## 5.4 Building Exterior

| TBD | Exterior Walls | ☒ Check if this section is NOT applicable. |  |  |
| --- | --- | --- | --- | --- |
| ☐ | Construction Type |  |  |  |
| ☐ | Condition |  |  |  |
| ☐ | U-Value |  |  |  |
| ☐ | Fire Rating |  |  |  |
| ☒ | North | Finish: | Material: | Thickness: |
|  |  | Height: | Lighting: | Other: |
| ☒ | South | Finish: | Material: | Thickness: |
|  |  | Height: | Lighting: | Other: |
| ☒ | East | Finish: | Material: | Thickness: |
|  |  | Height: | Lighting: | Other: |
| ☒ | West | Finish: | Material: | Thickness: |
|  |  | Height: | Lighting: | Other: |
| ☐ | Is there an awning or roof at Drive Thru window? |  |  |  |
| ☐ | Are exterior renovation included in scope of work? |  |  |  |
| ☐ | Is there Existing Building Signage? |  |  |  |
|  | Other: |  |  |  |

## 5.5 Roof

| TBD | Roof | ☒ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Assembly |  |
| ☐ | Type of water proofing |  |
| ☐ | Condition of roof |  |
|  | Structure |  |
| ☐ | framing size, type, and spacing |  |
| ☐ | Are structural modifications recommended/required? |  |
| ☐ | Slope of Roof |  |
|  | Other |  |

## 5.6 Columns

| TBD | Columns | ☒ Check if this section is NOT applicable. |  |  |
| --- | --- | --- | --- | --- |
| ☐ | Type / Size |  | Fire Proofing |  |
| ☐ | Typical Spacing |  |  |  |
| ☐ | Furring |  |  |  |

## 5.7 Demising Walls

| TBD | Demising Walls (Between Tenants) | ☒ Check if this section is NOT applicable. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ | Assembly |  | Fire Rating |  |  |
| ☐ | Condition |  |  |  |  |
| ☐ | Finish Material |  | Material Height |  |  |
| ☐ | Utilities? |  |  |  |  |
| ☐ | Sound Insulation |  |  |  |  |

## 5.8 Existing Interior Walls

| TBD | Existing Interior Walls | ☒ Check if this section is NOT applicable. |  |  |
| --- | --- | --- | --- | --- |
| ☐ | Assembly |  | Framing Height |  |
| ☐ | Condition |  |  |  |
| ☐ | Finish material |  | Material Height |  |
| ☐ | Are there (e) MEP Chases? Do any walls contain (e) utilities? |  |  |  |
| ☐ | Chases for MEP Systems |  |  |  |

## 5.9 Floors

| TBD | Floors | ☒ Check if this section is NOT applicable. |  |  |
| --- | --- | --- | --- | --- |
| ☐ | Assembly |  | U-Value |  |
| ☐ | Framing Type, Size, and spacing |  |  |  |
| ☐ | Fire rating |  |  |  |
| ☐ | Condition |  |  |  |
| ☐ | Is floor level? |  |  |  |
| ☐ | (e) Finish material |  |  |  |
| ☐ | Recommend for re-use |  |  |  |
| ☐ | Is existing flooring adequate for installation of new finish tile? |  |  |  |
| ☐ | Is a walk-off mat provided? |  |  |  |
| ☐ | Are structural modifications recommended or required? |  |  |  |

## 5.10 Ceilings / Soffit

| TBD | Ceilings | ☒ Check if this section is NOT applicable. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ | Suspended Ceiling |  |  | Height AFF |  |
| ☐ | Open to structure |  |  |  |  |
| ☐ | Finish Materials |  |  |  |  |
| ☐ | Soffits / Hardlid |  |  |  |  |
| ☐ | Finish Materials |  |  |  |  |
| ☒ | Hgt. to Bot. of Structure |  | Hgt. to Bot. of Sprinkler |  |  |
| ☒ | Hgt. to Bot. of Decking |  | Hgt. to Bot. of Roof Leader |  |  |
| ☒ | Hgt. to Bot. of Duct |  | Hgt. to Bot. of Diffuser |  |  |
| ☐ | Overall condition of ceiling |  |  |  |  |
| ☐ | Finish above dropped ceiling |  |  |  |  |
| ☐ | Features worth saving or incorporating into the design |  |  |  |  |

## 5.11 Lighting

| TBD | Lighting | ☒ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Interior lighting (type, heights, recommendations for reuse) |  |
| ☐ | Exterior lighting (type, heights, recommendations for reuse) |  |

## 5.12 Stairs

| TBD | Stairs | ☒ Check if this section is NOT applicable. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ | Construction Type |  |  |  |  |
| ☐ | Configuration |  | Handrail type |  |  |
| ☐ | Are Stairs part of Egress Route? Do they access a mezzanine? |  | Railing / Guardrail |  |  |
| ☐ |  |  | Handrail extensions |  |  |
| ☐ | Stair Width |  | Height |  |  |
| ☐ | Rise/Run |  | Tread Material |  |  |
| ☐ | Number of landings |  | Landing Clearance |  |  |
| ☐ | Are the stairs code-compliant? |  |  |  |  |

## 5.13 Restrooms

| TBD | Restrooms | ☒ Check if this section is NOT applicable. |  |
| --- | --- | --- | --- |
| ☐ | Configuration |  |  |
| ☐ | Fixtures/Hardware | Mens | Womens |
| ☐ | Lavatory Toilet/Urinal Partitions |  |  |
| ☐ | Fixtures Condition |  |  |
| ☐ | Are Fixtures compliant w/ ADA standards |  |  |
| ☐ | Restroom finishes |  |  |
| ☐ | Restroom Accessories |  |  |
| ☐ | Restroom Ceiling |  |  |

## 5.14 Security

| TBD | Security | ☒ Check if this section is NOT applicable. |
| --- | --- | --- |
| ☐ | Is there an existing security system? |  |
| ☐ | Is the system part of a larger system |  |
| ☐ | Security Sensors: |  |
| ☐ | Is there security lighting at the building exterior? |  |
| ☐ | Are there security grills or gates at building openings? |  |

# 6. FACILITY/JOB SITE MANAGEMENT

## 6.1 CONSTRUCTION

| TBD | Construction Information |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ | Permitted hours to work on premises? |  |  | Morning/Evening/Weekend/Hours |  |
| ☐ | What are delivery procedures during construction? |  |  | Describe |  |
| ☐ | What is the dumpster removal procedure during construction? |  |  | Describe |  |
| ☐ | What are portable toilet restrictions and placement options? |  |  | Describe |  |
| ☐ | What potential weather-related issues may affect construction and how? |  |  | Describe |  |
| ☐ | Are there any application or landlord-related criteria affecting construction? |  |  | Not applicable to this report. Due diligence report only. |  |
| ☐ | Is there a temp café planned at this location | ☐Yes ☒ No Not applicable to this report. Due diligence report only. | If Yes, describe due diligence required for temp. solutions? |  | Not applicable to this report. Due diligence report only. |

## 6.2 Other

| TBD | Other | ☒ Check if this section is NOT applicable. Not applicable to this report. Due diligence report only. |  |
| --- | --- | --- | --- |
| ☐ | Is there visible water damage in space? |  |  |
| ☐ | Is there an external storage space available? |  |  |
| ☐ | Is there any evidence of asbestos being present? |  |  |
| ☐ | Is there a fireplace? |  |  |
| ☐ | Is there an existing ice machine? Condenser Type? |  |  |
|  | Additional Information |  |  |